Blaise Gassend	Tuesday may 8th 2000

François-Marie Lefevere

###### DUST

##### Developer’s Universal SX Tool

User Manual

# Introduction

Dust is a SX micro-controller simulator and an electronic circuit simulator made to work together in order to make complex SX applications easy to debug.

The simulator has an interactive graphical environment for debugging. The electronic circuit that you want to debug is defined in a native language.

Before creating circuits it is recommended that you read the section of the Technical Manual that discusses the name space and the simulation algorithm.

If you are unsure of a word, try the glossary.

# File Format

General comments:

- The file is made up of definitions separated by semicolons.

The #include directive can be used to include external files.

- There are three different contexts in which definitions are accepted : The global context, the component definition context and the display box context.

- Except for components, symbols appearing in the global context are used to create startup windows.

- In the following, square brackets indicate optional items.

## Commenting your circuits

C++ style comments are used. No nested comments.

Valid examples :	/* ... */

/*  /*  */

// ....

Bad example : /* /* */ */

## Referring to a symbol in the namespace

To refer to a symbol you use the following notation :

PARENT1/PARENT2/.../PARENTn/NAME

The symbol is first searched for from the current location, then from the root of the tree, then from the main component.

In some cases (plugs and variables) it is possible to represent a whole range of symbols indexed by integers, using a compact notation :

Example : MEM/11~10/VAL/1~0

This is the same as MEM/11/VAL/1:MEM/11/VAL/0:MEM/10/VAL/1:MEM/10/VAL/0. This notation will be useful with plugs and variables.

Warning : There isn’t much checking about names with double definitions. If a name is defined twice the behavior is unpredictable. So avoid naming two symbols the same way, and if things seem really strange, try changing names.

## Including the contents of a file

Usage : #include ”filename”

Context : at the start of any line

Example : #include ”flop.circuit”

Includes the specified file. Can appear at the start of any line.

## Defining a component type

Usage : component NAME { CONTENTS };

Context : global

Example : component empty {};

Defines the component type NAME.

## Creating a component

Usage : COMPONENTTYPE NAME1[,NAME2[...]];

Context : component definition

Example : and and1, and2;

Inserts components of type COMPONENTTYPE with the names NAME1, NAME2, ...

## Connecting pins together

Usage : net [NAME] PIN1[-PIN2[...]];

Context : component definition

Example : net IN and1/IN-or2/IN;

Connects the specified pins together with a net of name net. The pins can only come from symbols under the current symbol to avoid having nets connected to themselves. There might be cases in which the program doesn’t detect that a net is referring to itself, try to avoid such situations.

Nets are considered to be pins, thus a macrocomponent can connect to the nets of its children.

## Inserting a symbol defined elsewhere

Usage : symbol [NAME] SYMBOL;

Context : global

Example : symbol OUT and1/out;

Inserts a copy of the specified symbol. Is often useful for displaying a symbol in a hbox or vbox, but also in a macrocomponent to mirror a constituent component’s symbol.

## Grouping symbols for display purposes

Usage :	hbox [NAME] { CONTENTS };

vbox [NAME] { CONTENTS };

Context : global, symbol definition or display box

Example : hbox { symbol AND1/OUT; symbol OR1/OUT; };

Groups the listed symbol horizontally (hbox) or vertically (vbox) for the display. Using hboxes and vboxes it is easy to create a good looking interface for any specific circuit.

## Grouping symbols for naming purposes

Usage : plug [NAME] <SYMBOL1[:SYMBOL2:[...]]>;

Context : global, symbol definition or display box

Example : plug A <AND4/STATUS:AND3/STATUS:AND2/STATUS:AND1/STATUS>;

In the preceding example it will now be possible to call AND3/STATUS by the name A/3. The following example shows that symbols put together in a plug can sometimes be named in a more flexible way :

Example : plug B <A/3~1>;

This notation is equivalent to plug B <A/3:A/2:A/1:A/0>;

Note that the numbers are in decreasing order. This is to be compatible with variables.

Note that it is possible to skip some numbers :

Example : plug P <A:B::C>;

In this case P/1 will be a nullsymbol.

In some cases it can be interesting to give names to the different elements of a plug instead of numbers. To do this you have to define a plugtype :

Usage : plugtype NAME <PATH1:PATH2:...:PATHn>;

Context : global, symbol definition or display box

Example : plugtype MYBUS <DATA/7~0:ADDRESS/15~0:READ:WRITE>;

This will define a plug type in which the symbols will bear names instead of numbers. To create a symbol on such a model use the following syntax:

Usage : PLUGTYPE [NAME] <SYMBOL1[:SYMBOL2[...]]>;

Context : global

Example : MYBUS MEMPLUG <MEM/DATA/7~0:MEM/ADDR/7~0:MEM/R:MEM/W>;

## Connecting many pins at a time with a bus

Usage : bus [NAME] PLUG1[=PLUG2[...]];

Context : component definition

Example : bus MAINBUS MEMPLUG=PROCESSORPLUG;

The various plugs must only be made up of pins. Corresponding pins will be connected. Note that the bus can later be used as a plug, just as a net can be used as a pin.

With bus it is only possible to connect plugs that are of exactly the same plug type. If you want to connect incompatible buses, you can use a mixedbus. Mixed buses are used in exactly the same way as busses between untyped plugs.

## Defining your own variables.

Usage : var [NAME] <BIT1[:BIT2[...]]>;

Context : global, symbol definition or display box

Example : var LENGTH <SCENIX/BANK_0/9~8>;

Creates a variable by putting together bits from other variables. It is possible only to take some bits from the variables :

Example : var FOO <BAR/4~3>;

Takes bits 4 and 3 from BAR for variable FOO.

Note that bits are specified with most significant bit first.

## This isn’t very flexible, there are plenty of other things I want to choose!

Actually, most of the other things you want to choose are very component specific, so there is a general way of specifying them called attributes, that can adapt to a wide variety of situations.

Usage : attribute1=value1[, attribute2=value2[...]];

Context : global, symbol definition or display box

Example : base = 10, Vcc = 6;

The specified attribute will apply to all symbols in the current context appearing after its definition. It will also apply to all the children of these symbols.

It is also possible to specify an attribute that only applies to one symbol, here is an example :

Example : variable FOO(base = 2) <BAR>;

This attribute will be propagated to all of FOO’s children in the symboltree. For hbox and vbox attributes that are specified here are not propagated to the children.

The values that an attribute can take are : integer, floating point, string, name and null. It is also possible for an attribute to take on the value of an attribute that was defined before it. In this way macrocomponents can receive parameters that they can distribute to their children.

Example : base = 2; Vcc = 5.234; title = ”Hello”; Vdd = ; child = AND1; f = %freq;

In this example, base will have the value 2, Vcc will have the value 5.234, title will represent the string ”Hello”, Vdd will be undefined, child will hold the name AND1 and f will contain the same value as freq.

It is possible to use metric multipliers when specifying floating point values. To do this, write an underscore followed by the multiplier you want. Valid multipliers are (in increasing order) : a, f, p, n, µ (-or u), m, c, d, (blank), da, h, k, M, G, T, P.

Example : f = 66_M

This sets the frequency to 66 Mhz.

## Examples

The Dust distribution contains a number of examples that should allow you to become more familiar with this language.

## Grammar

A full grammar of the language can be found at the end of this document if you want a more precise description.

# Linux Interface

The following section applies to the linux version only.

## Starting a session

Usage : xdust source1 [source2 [...]]

The program will parse the files in the order in which they appear. Only one file can contain a main component.

At startup, windows will be created for each symbol appearing in the global context. If no symbols are defined there, a window displaying the symboltree will appear.

Keep an eye open for error messages in the window you ran xdust from.

## Running your circuit

Most of what follows can be done with the keyboard. Have a look at the right mouse button menu.

### Resetting the circuit

The Reset button Resets the whole circuit.

### One simulation step at a time

With the One Tick button, one simulation step is executed.

### One SX instruction at a time

With the One Trace button, one SX instruction is executed. If there are many SXs, the first one to have finished an instruction stops the execution.

### Skipping over calls

With the One Step button, one SX instruction is executed, except that this time calls are skipped over. If there are many SXs, the first one to have finished a step stops the execution.

### Running continuously

There are many ways to run continuously. The simplest way is to use the Go button. The circuit is simulated without being updated. When you have had enough you can press Stop to stop.

If you want to have updates every once in a while, you can use the Fast button. The Fast speed edit box allows you to choose how many ticks go by between refreshes.

If you want to update after each Tick, Trace or Step, use the Animate buttons.

The Slow buttons leave a delay in miliseconds between Ticks, Traces or Steps that you can set with the Slow delay box.

### Running to the cursor

If you want to run until the cursor is reached, use the To Cursor button.

## Changing parameters

The right mouse button opens up a menu that allows you to interact with the selected symbol. Most of the functions that you find here have keyboard shortcuts that are indicated in the menu.

## Creating an interface interactively.

By clicking on the Edit Mode button of a window, you get a new toolbar that allows you to modify the interface interactively. Some of the actions you can do in edit mode are also possible in run mode, most are not.

### Selecting a symbol

When you move the mouse around the window, parts of it gets highlighted. The highlighted part is the selected symbol.

### Moving a symbol to another position

Use the middle mouse button to drag the selected symbol from one place to the other. The symbol has to be dropped on the lines that appear between adjacent symbols.

### Getting rid of a symbol

Drag the symbol onto the Trash Can button with the middle mouse button to get rid of it.

### Creating a new displaybox

Drag the New Horizontal or New Vertical display box button to the desired place with the middle mouse button.

### Inserting a project symbol

Drag the New Project symbol to the desired place with the middle mouse button.

### Creating a new window

The New Project, New Horizontal and New Vertical buttons create a new window when clicked on. The New Window From Selection button creates a new window with the selected symbol when you drag onto it with the middle mouse button.

### Saving a window

Once you have set up your interface the way you want it, you can export it using the Export button. You may have problems exporting anonymous symbols. Boxes will loose their names.

### Adding symbols from the project tree to your interface

You can add symbols from the project tree to your interface by dragging them with the left mouse button.

### Inserting a symbol by using its name

You can use the Insert Symbol edit box to type in the name of a symbol. The box is grayed out when the symbol name is invalid. When you press Enter, the program tries to complete the name you typed. Pressing enter many times cycles through the different possibilities.

Once a valid name is typed in, you can drag it to the desired location with the left mouse button.

# Windows interface

The following section applies to the Windows version only.

## Starting a session

Usage : windust [source]

The program will parse the source file if there is any specified.

At startup, the program will display the main window and additional windows will be created for each symbol appearing in the global context.

You can also open a source file from then Files|Open menu in the Main window of the application.

### Main Window

Once you have run Windust, this is the first windows that appears. A screenshot of this window if shown below:

The message box displays error messages that occurs when a file is parsed.

## Creating an interface interactively.

### Creating a new symbol view window :

This can be done from the Windows|New window menu in the Main window. You will obtain the window below:

This window contains a main box where symbols can be dropped. If this box is destroyed the window is destroyed immediately.

### Adding a new symbol :

There are two ways of adding symbols with the interface:

- using an insert symbol edit box.

Just type the name of the symbol you want to display in an insert symbol edit box, and drag it over a box or over another symbol. When you drop it, the new symbol is created and added in the window. (see the Moving a symbol section for more details about dropping symbols). If the name is invalid then an error message appears in the message box  of the main window.

- using a project widget.

Just select the symbol you want to display in the tree and drag it over a box or over another symbol. (see the Moving a symbol section for more details about dropping symbols)

### Creating a project widget :

A project widget is a widget containing a tree representing the hierarchy of the components, symbols and pins in the application.

If you type __project__ in an insert symbol edit box you will obtain, using drag & drop, a project widget containing the hierarchy of the whole simulation project.

If you type the name of a component that doesn’t export a specific viewer, you will obtain a project widget whose root will be that component. Some examples of project widgets are shown below :

### Creating a new horizontal/vertical box :

Horizontal and vertical boxes are fundamental widgets in the application. They are containers for all other widgets. A horizontal box puts all the widgets it contains in the same line while a vertical box will put all its widgets in a column.

To create a box, drag the horizontal or vertical box constructor button to the desired place.

Note : the type of the box (horizontal or vertical) can be modified after the creation of the box.

### Deleting a symbol :

Drag the symbol onto a Trash Can button.

Note : Deleting the main box of a window will destroy it.

### Moving a symbol :

Use the left mouse button to drag the symbol from one place to another.

When you drop the symbol onto a symbol other than a box, the symbol will be inserted after the symbol it is dropped on. When you drop the symbol over the title of a box, the symbol will be inserted in the box and will become the first item. If you drag the symbol over the bottom side or the right side of a box, the drag cursor will change of appearance. If you drop the symbol at this place, the symbol will be inserted after the box.

Insertion of a symbol after the totobibi2 box. The totobibi2 box is contained in a vertical box.

Insertion of a symbol after the totobibi2 box. The totobibi2 box is contained in a horizontal box.

### Modifying display parameters of a symbol :

Each symbol has its own popup menu accessible through a right click on the symbol. With this menu, it is possible to modify parameters like :

- the length of the symbol name (full name, short name ...)

- the type of value being displayed (byte, word , dword, signed, unsigned )

- the type of box (horizontal, vertical)

- the base of the value (binary, octal, decimal, hexadecimal )

### Modifying symbol sizes :

Symbols containing a small square at the bottom-right corner can be resized by dragging the square.

Boxes can be resized if the option Auto size in their popup menu is not checked.

Note : some sizes are imposed by the container :

- symbols contained in a vertical box will have their minimum width imposed by the minimum width of other symbols. To modify the width, you can modify the size of the container.

- symbols contained in a horizontal box will have their minimum height imposed by the minimum height of other symbols. To modify the height, you can modify the size of the container.

### Saving a window or a box :

You can export the text description of a box to the clipboard using the popup menu of the box.

The same operation can be done for a window, using the Window|Export to clipboard menu.

## Running a circuit.

### Execution control :

The execution of the simulation is controlled by the Simulation menu and by the buttons of the toolbar of a symbol view window.

Ticks runs one elementary step of the circuit simulation.

Trace will stop after the execution of each instruction of the micro-controller. (if there are several micro-controllers, the simulation will stop for each of them)

Step will stop after the execution of each instruction of the micro-controller but in case of a call, the simulation will stop after the return.

Run executes the simulation until the stop button is activated. Several refresh modes can be chosen in the Simulation|Refresh rate menu.

Note: The selection of the refresh mode can dramatically change the speed of the simulation.

### Animate mode :

You can follow the execution of your circuit at a slow run speed with the Simulate|animate menu. In this mode, the simulation kernel will be called periodically with a period of m milliseconds between each call. The constant m can be changed with the Simulate|Set animation speeds option through the Time by step edit box.

Three options are available:

- By step.

- By trace.

- By ticks.

These options are the animate versions of the step, trace and ticks options presented above.

To stop the animation : use the stop button.

### Refresh rate :

Three refresh rates are available in the Simulate|Refresh rate menu.

- Normal means that widgets will be refreshed every step.

- Fast animation means that widgets will be refreshed one time for n simulation ticks. The constant n can be changed with the Simulate|Set animation speeds option through the Steps before a refresh edit box.

- No refresh disables refresh.

### Animation parameters :

This dialog box is accessible with the Simulate|Set animation speeds menu. It allows you to configurate the animate mode and the running mode.

See the refresh rate section and the Animate section for more details.

### Breakpoints :

To set breakpoints, use the popup menu of disassembly widgets. At the present time, only simple breakpoints are available.

# Pin types

## Pin EDGE

### Description :

Input pin with detection of high to low or low to high transition. Specific pin used in the sx28 component. The pin configuration is only accessible by the program of the micro-controller.

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| STATE | VARIABLE | State of the pin : 0 or 1 |
| DETECT | VARIABLE | A transition has been detected if this bit is set. |

### Attributes :

None.

## Pin FLOATPIN

### Description :

General analog output pin with null impedance.

### Symbols :

None.

### Attributes :

None.

## Pin INCOMPPIN

### Description :

Universal logic input pin.

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| STATE | VARIABLE | State of the pin : 0 or 1 |

### Attributes :

| Name: | Type : | Default | Description : |
| --- | --- | --- | --- |
| Vcc | Real | 5 |  |
| Vdd | Real | 0 |  |
| Vtrig | Real | ##### Vcc+Vdd 2 | Voltage threshold between low level state and high level state. |

## Pin INPUT

### Description :

5V logic trigger input pin with infinite impedance. The low to high threshold is set to 1V and the high to low threshold is set to 4V.

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| STATE | VARIABLE | State of the pin : 0 or 1 |

### Attributes :

None.

## Pin OSCILLOINPUT

### Description :

Oscilloscope input . Impedance 10G Ohms.

### Symbols :

None.

### Attributes :

None.

## Pin OUTPIN

### Description :

Universal logic output pin.

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| STATE | VARIABLE | State of the pin : 0 or 1 |

### Attributes :

| Name: | Type : | Default | Description : |
| --- | --- | --- | --- |
| Vcc | Real | 5 | High level voltage. |
| Vdd | Real | 0 | Low level voltage. |
| Zout | Real | 0 | Output impedance. |

## Pin OUTPUT

### Description :

5V logic output pin with 25 ohms impedance.

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| LATCH | VARIABLE | State of the pin : 0 or 1 |

### Attributes :

None.

## Pin SCENIXPIN

### Description :

General IO pin for the SX micro-controller family.

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| STATE | VARIABLE | State of the pin as input : 0 or 1. |
| TRIS | VARIABLE | Direction of the pin: 1 = input, 0 = output. |
| LATCH | VARIABLE | State of the pin as output : 0 or 1. |
| PULLUP | VARIABLE | State of the pull-up resistor : 1= no pull-up, 0 = 20Kohms pull-up resistor. |

### Attributes :

None.

# Components

## Component and

### Description :

Two inputs AND gate.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| A, B | INCOMPPIN | Inputs. |
| O | OUTPIN | Output. |

### Symbols :

None.

### Attributes :

No attribute defined for the gate, but pin levels can be defined using pins attributes.

## Component clock

### Description :

Square clock.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| O | OUTPIN | Signal output. |

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| STATE | VARIABLE | State of the output pin : 0 or 1 |

### Attributes :

| Name: | Type : | Default | Description : |
| --- | --- | --- | --- |
| F | Real | 1E6 | Frequency of the clock. |
| T | Real | 1E-6 | Period of the clock. |

Note : Pin levels can be defined using pins attributes.

## Component flipflop

### Description :

Bistable flip-flop

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| S | INCOMPPIN | Set input. |
| R | INCOMPPIN | Reset input. |
| Q | OUTPIN | Output. |
| NQ | OUTPIN | Inverted output. |

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| STATE | VARIABLE | State of the output pin : 0 or 1 |

### Attributes :

No attribute defined for the flipflop, but pin levels can be defined using pins attributes.

## Component motor

### Description :

Linear motor driven by a PWM signal,  with incremental quadrature encoder.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| PWM | INCOMPPIN | Pulse Width Modulation input. |
| DIR | INCOMPIN | Direction input. |
| Q1 | OUTPIN | Quadrature output 1. |
| Q2 | OUTPIN | Quadrature output 2 |
| PosPin | FLOATPIN | Output pin giving a voltage corresponding to the position. |
| VelPin | FLOATPIN | Output pin giving a voltage corresponding to the velocity. |
| AccelPin | FLOATPIN | Output pin giving a voltage corresponding to the acceleration. |

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| Pos | REALPTR | Position. |
| Vel | REALPTR | Velocity. |
| Accel | REALPTR | Acceleration. |
| ExtForce | REALPTR | External force. |

### Attributes :

| Name: | Type : | Default | Description : |
| --- | --- | --- | --- |
| A, B | Real | 1 1 | Acceleration = A * STATE(PWM) * SIGN(DIR) - B * velocity |
| C | Real | 1 | Quadticks = x * C |
| Timestep | Real | .0001 | Step size for numerical simulation of motion equations. |

Note : Pin levels can be defined using pins attributes.

## Component nand

### Description :

Two inputs NAND gate.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| A, B | INCOMPPIN | Inputs. |
| O | OUTPIN | Output. |

### Symbols :

None.

### Attributes :

No attribute defined for the gate, but pin levels can be defined using pins attributes.

## Component nor

### Description :

Two inputs NOR gate.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| A, B | INCOMPPIN | Inputs. |
| O | OUTPIN | Output. |

### Symbols :

None.

### Attributes :

No attribute defined for the gate, but pin levels can be defined using pins attributes.

## Component not

### Description :

NOT gate.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| A | INCOMPPIN | Input. |
| O | OUTPIN | Output. |

### Symbols :

None.

### Attributes :

No attribute defined for the gate, but pin levels can be defined using pins attributes.

## Component or

### Description :

Two inputs OR gate.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| A, B | INCOMPPIN | Inputs. |
| O | OUTPIN | Output. |

### Symbols :

None.

### Attributes :

No attribute defined for the gate, but pin levels can be defined using pins attributes.

## Component oscillo

### Description :

Multi input oscilloscope. Any number of inputs can be used, just invoke a pin name starting with ‘_’. The oscilloscope starts sampling after a low to high transition on the Trigger input. It stops when the end of the memory is reached and restart when there is a new low to high transition on the Trigger input.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| CLK | INPUT | Sample clock. (Sampling on rising edge) |
| TRIGGER | INPUT | Trigger : start sampling after a low to high transition. |
| _XXXX | OSCILLOINPUT | Signal input pins. |

### Symbols :

This component exports a symbol which name is the same as the component name. The type of the symbol is OSCILLO.

### Attributes :

| Name: | Type : | Default | Description : |
| --- | --- | --- | --- |
| Memsize | Integer | 1024 | Number of samples that can be stored on each input. |

Note: pin levels can be defined using pin’s attributes

## Component powerdc

### Description :

Power supply.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| VCC | OUTPIN | Power output. |

### Symbols :

| Name: | Type : | Description : |
| --- | --- | --- |
| STATE | VARIABLE | Turn on/off the output of the power supply |

### Attributes :

No attribute defined for the gate, but pin levels can be defined using pins attributes. The output voltage can be configured using the following attribute:

| Name: | Type : | Default | Description : |
| --- | --- | --- | --- |
| Vcc | Real | 5 | Voltage of the output pin. |

## Component sx28

### Description :

8 bits RISC micro-controller.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| MCLR | INPUT | Master Clear reset input – active low |
| OSC1 | INPUT | External clock source input |
| OSC2 | OUTPUT | Not implemented yet. |
| RTCCPIN | EDGE | Input to Real Time Clock/Counter |
| RA0..RA3 | SCENIX_PIN | Port A bi-directional I/O pins. |
| RB0 | SCENIX_PIN | Port B bi-directional I/O pin; MIWU input; analog comparator Output |
| RB1 | SCENIX_PIN | Port B bi-directional I/O Pin; MIWU input; analog comparator negative input |
| RB2 | SCENIX_PIN | Port B bi-directional I/O pin; MIWU input; comparator Positive input |
| RB3..7 | SCENIX_PIN | Port B bi-directional I/O pins; MIWU inputs |
| RC0..RC7 | SCENIX_PIN | Port C bi-directional I/O pins. |

### Symbols :

| Name : | Type : | Description : |
| --- | --- | --- |
| STACK | ARRAY | Content of the stack used by the SX28 to manage CALL and RET functions. 2 or 8 levels, depending of the value of Fuse and Fusex. |
| ROM | ARRAY | Content of the program ROM. (not yet implemented on Linux platform) |
| BANK_0 | ARRAY | Registers from 0x08 to 0x0F. |
| BANK_1 | ARRAY | Registers from 0x10 to 0x1F. |
| BANK_3 | ARRAY | Registers from 0x30 to 0x3F. |
| BANK_5 | ARRAY | Registers from 0x50 to 0x5F. |
| BANK_7 | ARRAY | Registers from 0x70 to 0x7F. |
| BANK_9 | ARRAY | Registers from 0x90 to 0x9F. |
| BANK_B | ARRAY | Registers from 0xB0 to 0xBF. |
| BANK_D | ARRAY | Registers from 0xD0 to 0xDF. |
| BANK_E | ARRAY | Registers from 0xF0 to 0xFF. |
| Code | DISASSEMBLY | Disassembly window : display only lines containing opcodes. Lines are sorted by increasing ROM addresses. |
| Source | DISASSEMBLY | Disassembly window : display all the lines contained in the lst file. Lines are sorted by increasing number |
| PC | VARIABLE | Program Counter. |
| FUSE, FUSEX | VARIABLE | Registers containing the configuration of the SX. |
| C | VARIABLE | Carry bit. |
| DC | VARIABLE | Digital Carry bit. |
| Z | VARIABLE | Zero bit. |
| PD | VARIABLE | Power down bit. |
| TO | VARIABLE | Watch Dog Timeout bit. |
| PAGE | VARIABLE | Program memory page selection bits. |
| W | VARIABLE | Working register |
| INDF | VARIABLE | Indirect Addressing through FSR : the register location pointed to by FSR is accessed. |
| RTCC | VARIABLE | Real-Time Clock/Counter. |
| MODE | VARIABLE | Mode register |
| OPTION | VARIABLE | Option register |
| STATUS | VARIABLE | Contains the device status bits like C,Z,DC |
| FSR | VARIABLE | File Select Register. |
| RA, RB, RC | VARIABLE | Value of ports A,B,C inputs. |
| TRISA, TRISB, TRISC | VARIABLE | Direction of ports A, B, C |
| R_OUTA, R_OUTB, R_OUTC | VARIABLE | Value of ports A, B, C outputs |
| PLPA, PLPB, PLPC | VARIABLE | Pull-ups of ports A, B, C |

### Attributes :

| Name : | Type : | Default | Description : |
| --- | --- | --- | --- |
| File | String | - | Name of the code file: supports both .HEX files and .LST files |
| Fuse | Integer | File value | Value of the Fuse configuration word. |
| Fusex | Integer | File value | Value of the Fusex configuration word. |

## Component xnor

### Description :

Two inputs XNOR gate.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| A, B | INCOMPPIN | Inputs. |
| O | OUTPIN | Output. |

### Symbols :

None.

### Attributes :

No attribute defined for the gate, but pin levels can be defined using pin’s attributes.

## Component xor

### Description :

Two inputs XOR gate.

### Pins :

| Name: | Type : | Description : |
| --- | --- | --- |
| A, B | INCOMPPIN | Inputs. |
| O | OUTPIN | Output. |

### Symbols :

None.

### Attributes :

No attribute defined for the gate, but pin levels can be defined using pin’s attributes.

# Symbols

## Symbol ARRAY

### Description :

Array of integer values.

### Attributes :

| Name: | Type : | Linux | Windows | Default | Description : |
| --- | --- | --- | --- | --- | --- |
| base | Integer | X | X | 10 | Base : 2 = binary , 8 = octal,10 = decimal, 16 = hexadecimal ... |
| columns | Integer | X | X | 4 | Number of columns. |
| datawidth | Integer | X | X | 8 | 8 = bytes, 16 = 16-bits words, 32 = 32-bits wordsl |
| disptitle | String | X | X | - | title |
| fullname | Boolean | X | X | 0 | 0 = short name, 1 = full name |
| height | Integer |  | X | 169 | Height of the array widget in pixels. |
| reduced | Boolean |  | X | 0 | 0 = normal display mode, 1 = reduced mode (no title). This mode is used to put arrays together (example : to see the SX register memory) |
| signed | Boolean | X | X | 0 | 0 = unsigned, 1 = signed. |
| title | Boolean | X | X | 1 | 0 = no title, 1 = display title |
| width | Integer |  | X | 257 | Width of the array widget in pixels. |

## Symbol DISASSEMBLY

### Description :

Displays disassembly code, breakpoints position and instruction pointer position.

### Attributes :

| Name: | Type : | Linux | Windows | Default | Description : |
| --- | --- | --- | --- | --- | --- |
| disptitle | String | X | X | - | title |
| Fullname | Boolean | X |  | 0 | 0 = short name, 1 = full name |
| Height | Integer |  | X | 153 | Height of the disassembly widget in pixels. |
| Title | Boolean | X | X | 1 | 0 = no title, 1 = display title |
| Width | Integer |  | X | 265 | Width of the disassembly widget in pixels. |

## Symbol HBOX

### Description :

Horizontal widget container.

### Attributes :

| Name: | Type : | Linux | Windows | Default | Description : |
| --- | --- | --- | --- | --- | --- |
| Disptitle | String | X | X | - | title |
| Fullname | Boolean | X | X | 4 | 0 = short name, 1 = full name |
| Title | Boolean | X | X | 1 | 0 = no title, 1 = display title |

## Symbol REALPTR

### Description :

Displays a real value.

### Attributes :

| Name: | Type : | Linux | Windows | Default | Description : |
| --- | --- | --- | --- | --- | --- |
| Disptitle | String | X | X | - | title |
| Fullname | Boolean | X | X | 4 | 0 = short name, 1 = full name |
| Title | Boolean | X | X | 1 | 0 = no title, 1 = display title |

## Symbol OSCILLO

### Description :

Displays evolution in time of logic or analogic signals.

### Attributes :

| Name: | Type : | Linux | Windows | Default | Description : |
| --- | --- | --- | --- | --- | --- |
| disptitle | String | X | X | - | title |
| fullname | Boolean | X | X | 0 | 0 = short name, 1 = full name |
| height | Integer |  | X | 177 | Height of the disassembly widget in pixels. |
| title | Boolean | X | X | 1 | 0 = no title, 1 = display title |
| width | Integer |  | X | 257 | Width of the disassembly widget in pixels. |

## Symbol VARIABLE

### Description :

Displays integer value.

### Attributes :

| Name: | Type : | Linux | Windows | Default | Description : |
| --- | --- | --- | --- | --- | --- |
| base | Integer | X | X | 10 | Base : 2 = binary , 8 = octal,10 = decimal, 16 = hexadecimal ... |
| disptitle | String | X | X | - | title |
| fullname | Boolean | X | X | 0 | 0 = short name, 1 = full name |
| signed | Boolean | X | X | 0 | 0 = unsigned, 1 = signed. |
| title | Boolean | X | X | 1 | 0 = no title, 1 = display title |

## Symbol VBOX

### Description :

Vertical widget container.

### Attributes :

|  | Type : | Linux | Windows | Default | Description : |
| --- | --- | --- | --- | --- | --- |
| disptitle | String | X | X | - | title |
| fullname | Boolean | X | X | 4 | 0 = short name, 1 = full name |
| title | Boolean | X | X | 1 | 0 = no title, 1 = display title |

## Other symbols

### __project__

Symbol that represents the whole hierarchy of the project.

### __time__

Real value giving the simulation time.

# Glossary

pin : A component has a number of electrical contacts that are used to connect it with neighboring components. These contacts are pins. They are characterized by their input voltage, their output voltage and their output load.

component : The elementary building block of an electronic circuit.

macrocomponent : Component that is made of other components.

net : Electrical connection between a number of pins.

widget : Building block of a graphical interface (buttons, editing fields, but also boxes that can contain a number of widgets).

attribute : A name associated with a value that can be used to modify a symbol or a widget. The attribute class stores a list of attributes terminated by a nullattribute.

nullattribute : Value that is used to pass an absence of attributes.

symbol : Element of the namespace.

variable : Collection of bits that can be visualized or modified together.

bus : Connection between plugs that connect the pins of one plug with corresponding pins of the other plugs.

plug : Initially a group of pins that can be connected to other plugs with a bus. Was generalized to be able to group any type of symbol in the namespace in a convenient way.

symbolconstructor : Object that stores the characteristics of a symbol before it can be built.

constructor : See constructor.

componenttype : Object used to call the constructor of a component. Mainly useful for macrocomponents.

macrocomponent : Object used to store the characteristics of a macrocomponent and construct it when it is needed.

real : typedef that is used for floating point values. We used double, but this is probably an overkill.

SX : High speed micro-controller from Scenix that this is all about.

Scenix : The guys who developed the SX.

symbolspace : A tree made with symbols at its nodes and its leaves that contains everything you can (and want?) to manipulate.

namespace : An imaginary tree that bears the names of the symbols in symbolspace.

name : Identifies the children of a symbol in symbolspace. Is represented by an int that is associated with a string by the symboltable.

symbolname : Identifies a symbol with a relative or an absolute offset. Is represented by a list of integers and displayed as name1/name2/name3.

symboltable : Class that stores a table of strings that are frequently used and that are usually the names of symbols or attributes. The strings are associated with integers. Positive or zero integers represent themselves, even negative integers represent a string, odd negative integers are for names that weren’t specified (unnamed).

list : Class that implements a simple linked list.

# The grammar

Here is the full grammar that is used. It might help clarify details that I may have forgotten.

circuit : globaldefs EOF

globaldefs :

| globaldef SEMICOLON globaldefs

| SEMICOLON globaldefs

globaldef : componenttype

| boxsymbol

| plugtype

| attributes

componenttype : COMPONENT NAME LEFT_CURLY componentdefs RIGHT_CURLY

componentdefs :

| componentdef SEMICOLON componentdefs

| SEMICOLON componentdefs

componentdef : netsymbol

| componentsymbol

| plugsymbol

| bussymbol

| boxsymbol

| varsymbol

| symbolcopysymbol

| attributes

componentsymbol : NAME componentconstructorlist

componentconstructorlist : componentconstructor

| componentconstructor COMMA componentconstructorlist

componentconstructor : symbolname

boxsymbol : HBOX LEFT_CURLY boxdefs RIGHT_CURLY

| VBOX LEFT_CURLY boxdefs RIGHT_CURLY

| HBOX symbolname LEFT_CURLY boxdefs RIGHT_CURLY

| VBOX symbolname LEFT_CURLY boxdefs RIGHT_CURLY

boxdefs :

| boxdefs boxdef SEMICOLON

| boxdefs attributes SEMICOLON

boxdef : boxsymbol

| symbolcopysymbol

| plugsymbol

| varsymbol

plugsymbol : PLUG $$1 LEFT_POINTY namelist RIGHT_POINTY

| PLUG $$2 symbolname LEFT_POINTY namelist RIGHT_POINTY

| NAME LEFT_POINTY $$3 namelist RIGHT_POINTY

| NAME symbolname LEFT_POINTY $$4 namelist RIGHT_POINTY

plugtype : PLUGTYPE NAME LEFT_POINTY namelist RIGHT_POINTY

varsymbol : VAR LEFT_POINTY namelist RIGHT_POINTY

| VAR symbolname LEFT_POINTY namelist RIGHT_POINTY

bussymbol : BUS pluglist

| BUS symbolname pluglist

pluglist : composedname

| composedname EQUAL pluglist

symbolcopysymbol : SYMBOL symbolname composedname

| SYMBOL composedname

netsymbol : NET pinlist

| NET symbolname pinlist

pinlist : composedname

| composedname MINUS pinlist

namelist : namelistdef

namelistdef : namelistsymbolname

| namelistsymbolname COLON namelistdef

namelistsymbolname :

| NAME

| NAME SLASH namelistsymbolnametail

namelistsymbolnametail : extendedname

| range

| extendedname SLASH namelistsymbolnametail

| range SLASH namelistsymbolnametail

range : INTEGER TILDA INTEGER

composedname : NAME

| NAME SLASH composednametail

composednametail : extendedname

| extendedname SLASH composednametail

extendedname : NAME

| INTEGER

symbolname : NAME

| NAME LEFT_PAREN attributes RIGHT_PAREN

attributes : attribute

| attribute COMMA attributes

attribute : NAME EQUAL INTEGER

| NAME EQUAL REAL

| NAME EQUAL composedname

| NAME EQUAL STRING

| NAME EQUAL PERCENT NAME